Blaine McMahon, Jacob Sword, Nicholas Yameen

Network Design

Project Phase 5

4/21/19

1. Introduction:

This code completes Phase 5 of the project by using UDP to send images from a client to a server, and vice versa across a channel that has the possibility to introduce bit errors and lost packets.  This version utilizes Go-Back-N protocol to communicate across the channel.  The UDP server runs continuously to accept messages from clients.  These messages are in the form of:

1. A filename of the image, that the client will send

2. The contents of said image.

The server then writes the file.  After sending this file, the client waits for a response of the same nature from the server, and then writes it’s transferred file before exiting.  The below flowcharts and explanations explain these procedures in more detail.

2. Flowcharts

Fig 1. Server Flowchart

Fig 2. Client Flowchart

GUI.py

This file is the main GUI for the program. It uses tkinter to create the GUI. First, the GUI window will open and display three buttons, three input boxes, and two checkboxes. The user will first enter their intended file to send in the first input box, or leave it blank to use our default provided file path. Also, the user can now enter which checkboxes they want which correspond to which printouts are displayed.  Checking “Status Msgs” will allow messages such as “Server: Finished writing received file server_recv_spongebob.jpg” to be printed to the console.  Checking “UDP Err Msgs” will allow messages such as “ACK Packet Dropped!” to be printed to the console.  The user then has to click Start Server. This will start the Server function shown below. This function gets the entered filepath, or uses the default, and instantiates a UDPserver class with the noted filepath, and status_msgs.get() which gets the state of the “Status Msgs” checkbox.

The server class’s “listen” function is then run in a new thread.  Next, the user can enter their desired “Corruption Option” and “UDP Error Rate” in the corresponding input boxes of the GUI. The, the user can click the “Start Client” button.  This will call Client function shown below.

This function operates in a similar fashion to the above-detailed Server function, and the client’s start_send function will run in a new thread.  However, before this is done, apply_new_settings() is called.  This function gets the data from the “Corruption Option” and “UDP error rate” input boxes, in addition to the checkboxes, and passes this data onto our config.py file, so that it can be later accessed by functions in the rdt_sender.py and rdt_receiver.py files.

UDPserver.py

In the UDP server file, there is a class called UDPserver. The init function will first either use the file path given by the user at the start or use the default if not specified by the user. It will then open a UDP socket and bind to it and make it an instance variable.

When called, the listen() function will continually run, where it will listen for incoming connections and starts the RDT receive sequence

It will then preform a rdt_rcv(), utilizing the rdt_receiver.py file, where it will continually receive packets from the client and write them to the file until there are no packets left, in a Go-Back-N manner. Once the full image is received it will then close the file and send an image back to the client.

Messages will be printed to the console through this to show the status.  All messages printed from the server are run through self.print() to prefix it with the “Server:” string for easier reading in the terminal.  However, this is only done if the checkbox for status_msgs (passed in when thread was started) was checked.

The send_img() function will first open the file specified or the default one for reading binary. Finally, it will send the image using the using the rdt_sender.py function rdt_send().

UDPclient.py

The UDPclient has a class which defines four functions. First, the client will initialize similar to the UDP server. It will connect to the server on the UDP socket, which is made into an instance variable

Then it will go into start_send() function. It will first call send_img() function to send the server an image.

The send_img() function will first either use the path given by the user or the default one.  If there is an error finding the given filename, it will be caught above, and the client will stop execution. If the file is successfully found, it will open that image for reading binary. The client preforms RDT send by calling the rdt_send() function in the rdt_sender.py file. Once it has finished it will close that file.

Next, the start_send() will call the wait_and_receive() function. Here the client will receive the response image name from the server. It will open that file for writing binary. After which it will call rdt_rcv() (from the rdt_receiver.py file) to receive all the packets of the image. After which, the file will close, and the elapsed time will be printed to the console, which is later used in data collection to constructor our graphs.  The client also uses a self.print function to prepend “Client:” to all print strings.

rtd_sender.py

In the file, there are 5 functions defined. The server and client will use these functions to send Go-Back-N over a socket. The sender will use rdt_send() function. This function will take in a file, endpoint, and the socket. It first starts by making a buffer to hold the packets configured with a set window size and buffer capacity. During this time, it also configures and initializes the timer. Before the packets can be added to buffer a separate thread is started to ensure the receiver is listening.  This is one of the main distinctions in phase 5.  We have a separate thread running to ensure when packets are waiting to be received, it doesn’t block the main thread, sending the packets.

After the initial configurations have been done the loop condition is checked, that is, if the current packets sequence number is less than the base plus the window size. This is to ensure we are not sending packets that are not in the current window. The packets are then made and added to the buffer.  The current window is then specified by the window size.  These packets are then sent, and the timer is started.  If the timer times out, then all the previous packets that were not ACK’d will be resent using Go-Back-N protocol.  However, if a packet is received that is not corrupt and, it will carry on.  If the received packet is corrupt or the wrong sequence, the sender will do nothing. It will continuously make and send packets to the receiver in this fashion, each iteration until the file is completely sent, using the same timer logic each time.  Additionally, data packet loss is implemented on this side.  As shown below, if the user-defined corruption-option allows for data loss, and the random channel value is lower than the given present corrupt, the packet will not be sent, and will move on through the while loop.

For the timer logic, a timer class RDTTimer is used (from rdt_utils.py).  This timer class allows for “resetting” a timer by creating a new instance of it, and passing a callback function to call in the case of a timeout.  For the callback function, we use the inner-defined timeout_func function, which is shown above.  This function returns a function the prints a statement to the screen (if the “debug” config option is set, which is controlled by the UDT Error Rate checkbox in the GUI), then resends the current packet and restarts the timer.  A nested function was needed for this, as each instantiation of this function will have a different value for the packet parameter.  Note, the instantiation of the timer class above uses the timeout value configurated in config.py,this default value is 60ms.  The Timer class utilizes Threading.Timer, and is shown below.

A class needed to be used for this, rather than a simple instantiation of Threading.Timer, because Python does not allow the re-assigning of variables from upper scope within a function.  As such, we would not have been able to reassign timer to a new timer, which needs to be done after a timeout, and we had to rely on the above shown RDTTimer.start() function to do it instead.

To make packets, the make_pkt() function is used to read 1024 bytes at a time. If there is nothing to read, then it will return 0. This is so the rdt_send() can break out of the while loop. It takes the data read and the sequence number to calculate a checksum number. Finally, it creates the packet to be sent.

To calculate the checksum, the function calc_checksum() will be called. This preforms the 1’s complement of wraparound 16-bit sum.

The udt_send() function will send the packets to the endpoint over the socket. It returns the number of bytes sent.

The rdt_rcv() function will receive the acknowledgement from the receiver. It will be called with the socket and sequence number being sent. It will first extract the ACK from the receiver using extract(). This function will wait for a packet, if it doesn’t receive something it will timeout.

If it did receive a packet it will parse that packet.

It will parse the checksum from the data using the parse_checksum() function.

It then creates its own checksum, this is so the received and calculated checksums can be compared for corruption. Finally, it will check to see if the parsed packet isn’t corrupt, if not it returns 1.

To corrupt the data on the sender side, the function rdt_rcv() will corrupt the ACK. In the if statement the integer should be a value between 0 (no corruption) to 60 (max corruption). This will have to be changed by the user, as noted above.  There are two functions to pick a number between 0-100 called random_channel(). This number determines the percentage. The data will be corrupt in the corrupt_bits() function.

Additionally, packet loss is implemented here using the same logic, if the given corruption open is used.  The Boolean value of corruption options states are defined by the given has*err() functions in rdt_utils, shown below.

rdt_receiver.py

The receiver will utilize the functions inside the rdt_receiver.py to complete Go-Back-N  transactions. The first function is rdt_rcv(), which takes the file to write to, the endpoint, and the socket. This function will continuously run until there is no packet received. It will first setup initial parameters.

The receiver will extract a packet and parse the packet accordingly. If it is the first packet it, it will open a new file for writing binary.

The extract() function will receive a packet from the socket. If there is a packet it will return it, otherwise it will return 0.

After, the packet is parsed as seen above. The received sequence number and data are stripped from the packet. The received checksum is parsed through the parse_checksum() function.

If the user selected some type of corruption, it will notify the user if there is corruption and corrupt the data. Otherwise it will just make its own checksum.

To verify the data is not corrupt, the data and sequence number received will be made into a checksum using the calc_checksum() function.

The expect sequence number, received sequence number, calculated checksum and received checksum are compared.

If the everything matches it will deliver the data using the deliver_data() function. This function writes the data to the file.

Then it will make an acknowledge packet using the make_pkt() function. This function will make a new checksum and create the ACK packet.

The packet will then be sent over the socket back to the sender.

The expected sequence number will then increase and the once through variable will be set. If there was a corrupt packet, the previous ACK packet will be sent again. Finally, if there was no packets left to receive, the file will close and the rdt_rcv() will return.

On the receiver side, rdt_rcv() can corrupt the data bits, similar to as before.

As said before, the integer value will have to change to a number between 0 – 60. In the image above it is 10% corruption.

There are two functions to pick a number between 0-100 called random_channel(). This number determines the percentage. The data will be corrupt in the corrupt_bits() function.

rdt_utils.py

This file holds all the functions that both the receiver and sender use. The first function is to corrupt data. Using some rando number, certain bits will be changed.

Next is the calc_checksum function. This function was shown in the previous sections. However, it preforms the 1’s compliment addition to create a checksum.

The next function if a function used to translate a decimal number to binary, called seq_num_to_bin().

New to Phase 5 is the PacketBuffer class. This class defines 6 functions. The init function will set the next sequence number, base, and window size. The rest of the function define what to do for the window buffer. First, cur_window() returns all packets from base to next sequence number. The add() function adds new packets to the window. The cur()function will return packets at next sequence number. Equal_index() function will return when base is equal to next sequence number. Lastly, ready() returns when the sender is ready to send more packets.

Config.py

The last file is our config file. This file holds all of out configurations to preforms Go-Back-N. The contents of this file are below.

How to Run

- Make sure all files laid out in the ReadMe.txt are present.

- Issue the command “python GUI.py” on the command line. The following GUI will appear.

- Leave file to send blank to use default or specify path to your own picture. Click server

- Click Status Msgs, if you would like to see messages such as “Client: Sending filename spongebob.jpg to server “, and click “UDP Err Msgs” if you would like to see error status messages like "Data Packet Dropped!".  Enter the desired corruption option (1-5).  Entering 1 denotes no errors, 2 denotes bit error in ACK packets, 3 denotes bit error in Data packets, 4 denotes loss of ACK packets, and 5 denotes loss of Data packets.  These options will be used by both client and server.

- Again, leave image blank to use default, or specify a different image, this time for the client to send. Then click client.

- the terminal messages will appear (if relevant checkboxes are clicked) of what is happening as seen below.

- At the end of the bidirectional transfer, the client will print out the completion time for the transfer.  This printout happens whether or not the checkboxes are clicked.

- New files will be prefixed with “client_recv” and “server_recv” depending on which entity they were received by.

- Click client again to re-run the client or click close to exit.  Entering a new image to send, or corruption/debug options will be applied when client is run again.

- To see what would happen if there was no loss recovery, uncheck the Loss recovery box and run the server and client with errors of some type. An example is below.



Server received errors:

Client received

Results:

The below images show test output of the program using a ~1MB file (spongebob.jpg, included), and corruption options 4 and 5 respectively with 20% error rate.  “Status Msgs” and “UDP Err Msgs” output was enabled for these images.  (Note: full output to completion of transfer was not included as it was many pages long).

Test Output, Corruption Option 4 with 20% Error Rate

Test Output, Corruption Option 5 with 20% Error Rate

Finally, to test the result of different corruption options and error percentages, tests were run to compare time-to-finish the bidirectional image transfer at increments of 5% error rate for corruption options 2 through 5, and compared with a control timing that was conducted with corruption option 1.  For these tests, the same ~1MB jpg file noted above was used, and all debugging print statements were turned off.  Also, the default timeout value of 60ms (defined in config.py) was used.  From these results, a plot was created.  Each data-point in the plot represents the average of 3 trial runs at the given bit error rate.  This plot can be seen below.

From this plot we see that Go-Back-N handles No Corruption, Bit Errors in the ACK, and ACK Packet Loss the best. For Option 1, Option 2, and Option 4, the image appeared to be sent in about 12s for each percentile. The slope is nearly zero for each. This is because Go-Back-N handles ACK packet errors best because of the collective ACK. If ACK packets are wrong the sender will send them again, only to have the last ACK’d packet to be sent again. This mean the sender doesn’t need to handle making new packets. However, for Data Loss (option 5) and Data Error (option 3), Go-Back-N seems to not handle it well. It took more time to complete the longer it took. The sender would have to send the wrong packet again as well as any left in the window. This is what made the time so long. The slope for Option 3 was 1.57 and Option 5 slope was 1.28. To conclude, Go-Back-N handles ACK loss the best due to the fact it is a collective ACK.

The next graph is the Time to Complete with a fixed loss rate of 10% while varying the window size. Again the trail was run 3 times each. Below is the results.

Here we see a small window size (less than 2) has varying results. However, looking at the data, each packet roughly took the same time to transfer given its Option. When comparing it to graph 1, the times at 10% Loss/Error are roughly the same. This shows that a varfyed window does not affect its outcome. The optimal window size can be any size greater that 2.

The last chart is comapring Phase 3, Phase 4, and Phase 5.

Here we see that Phase 4 has the more linear graph, averaging around 15s to complete. However, when it comes to ACK Loss or Error, Phase 5 completes its transfer faster by about 2 seconds. Phase 3 appears to be somewhere between 4 and 5. To conclude, Phase 5 is best for ACK Loss or Errors, while Phase 4 is best for Data loss.